WINCH SYSTEM PERFORMANCE & THERMAL REPORT (TRIAL 3)

Document Reference: Winch-MJ35-VALIDATION-TRY3
Test Date: July 1, 2026
Load: 20 Tons
Total Cycles: 20 Cycles

# 1. Executive Summary

This report provides a formal evaluation of the Isoloader MJ35 winch system during the third trial (Try 3). The trial consisted of 20 complete raise and lower cycles with a 20T load, performed continuously over 44.93 minutes. Telemetry logs indicate consistent regenerative energy recovery (43.83%) and stable motor heating rates. The maximum motor temperature reached was 73.0°C (Winch C), confirming that the tời operates safely within target limits.

# 2. Hoisting Speed Analysis

Lifting speed was calibrated using the stroke integration method. The average maximum raising speed was 6.74 m/min (with a peak log speed of 7.07 m/min), and the average maximum lowering speed was 6.00 m/min. The average stroke speeds were 5.64 m/min (raise) and 4.12 m/min (lower). The results confirm compliance with the 6.0 m/min laden specification.

# 3. Energy Consumption & Regeneration

Total energy consumption and battery capacity projections are modeled below based on the high-voltage pack logs:

• Gross Energy Discharged: 6.2549 kWh (0.3127 kWh/cycle)
• Regenerated Energy Recovered: 2.7414 kWh (0.1371 kWh/cycle)
• Net Energy Consumed: 3.5134 kWh (0.1757 kWh/cycle)
• Regeneration Percentage: 43.83%
• Projected Cycles on 80% SOC (91.62 kWh): 292.9 cycles (gross) / 521.5 cycles (net)

# 4. Motor Thermal Performance

| Winch Motor | Start Temp | End Temp | Max Temp | Temp Rise | Heating Rate |
| --- | --- | --- | --- | --- | --- |
| Winch A | 40.0°C | 68.0°C | 71.0°C | +31.0°C | 0.69°C/min |
| Winch B | 0.0°C | 0.0°C | 0.0°C | 0.0°C | N/A (Fault) |
| Winch C | 42.0°C | 70.0°C | 73.0°C | +31.0°C | 0.69°C/min |
| Winch D | 40.0°C | 67.0°C | 67.0°C | +27.0°C | 0.60°C/min |

All motor temperatures remained safely below the 80.0°C safety threshold and the 85.0°C critical warning limit. The average heating rate of 0.69°C/min (Winch C) was lower than Trial 2 due to a lower initial baseline (42.0°C) and a slightly lower electrical load per cycle, demonstrating that pre-cooling allows longer continuous runs.

# 5. Performance & Energy Trend Plot