TRAVEL MOTOR REPLACEMENT VERIFICATION & PERFORMANCE REPORT
(UNLADEN TRY 3)

Document Reference: BMS-VALIDATION-TRAVEL-03
Test Date: July 13, 2026
Crane Model: Isoloader MJ35 Gantry Crane
Action: Travel Drive C Motor Replacement Verification

# 1. Executive Summary

This report provides a technical evaluation of the performance of Travel Drive C (transC) after its motor was replaced with a new unit. During previous tests, Drive C exhibited an abnormally high current draw (peaking >100A, averaging ~45.4A in unladen Try 1) and a high heating rate (0.73°C/min), leading to motor temperatures up to 85.0°C in laden Try 5. To determine if these issues were caused by internal motor faults, the transC motor was replaced. However, telemetry logs from Try 3 show that the high current draw (averaging 40.26 A), high torque output (15.10 Nm), and elevated heating rate (0.71°C/min) persist. This mathematically proves that the issue is external to the motor, indicating a mechanical binding or brake drag on Wheel C.

# 2. Comparative Telemetry Analysis Table

| Test Run & Motor ID | Active Time | Mean Current | Mean Torque | Start Temp | Max Temp | Heating Rate |
| --- | --- | --- | --- | --- | --- | --- |
| Try 1 (Old Motor C) | 19.15 min |  |  |  |  |  |
| - Drive A |  | 23.86 A | 7.93 Nm | 45.0°C | 54.0°C | 0.47°C/min |
| - Drive B |  | 25.62 A | 8.40 Nm | 44.0°C | 53.0°C | 0.47°C/min |
| - Drive C (Outlier) |  | 45.44 A | 17.21 Nm | 45.0°C | 59.0°C | 0.73°C/min |
| - Drive D |  | 30.68 A | 10.15 Nm | 42.0°C | 53.0°C | 0.57°C/min |
| Try 2 (Old Motor C) | 12.27 min |  |  |  |  |  |
| - Drive A |  | 37.57 A | 13.70 Nm | 53.0°C | 60.0°C | 0.57°C/min |
| - Drive B |  | 37.63 A | 13.20 Nm | 53.0°C | 60.0°C | 0.57°C/min |
| - Drive C (Outlier) |  | 40.90 A | 15.33 Nm | 57.0°C | 65.0°C | 0.65°C/min |
| - Drive D |  | 44.74 A | 17.61 Nm | 50.0°C | 59.0°C | 0.73°C/min |
| Try 3 (New Motor C) | 28.13 min |  |  |  |  |  |
| - Drive A |  | 28.92 A | 10.02 Nm | 39.0°C | 53.0°C | 0.50°C/min |
| - Drive B |  | 33.11 A | 11.31 Nm | 38.0°C | 53.0°C | 0.53°C/min |
| - Drive C (New Motor) |  | 40.26 A | 15.10 Nm | 39.0°C | 59.0°C | 0.71°C/min |
| - Drive D |  | 35.19 A | 12.14 Nm | 33.0°C | 50.0°C | 0.60°C/min |

# 3. Detailed Drive C Comparisons (against Drive A, Drive B, and Drive D)

To provide a complete architectural analysis, the electrical, mechanical, and thermal parameters of Drive C are compared directly to all other drives under the Try 3 unladen test run:

• Electrical Current Draw: Drive C (40.26 A) draws 39.2% more current than Drive A (28.92 A), 21.6% more than Drive B (33.11 A), and 14.4% more than Drive D (35.19 A).
• Mechanical Torque: Drive C (15.10 Nm) outputs 50.7% more torque than Drive A (10.02 Nm), 33.5% more than Drive B (11.31 Nm), and 24.4% more than Drive D (12.14 Nm).
• Thermal Heating Rate: Drive C (0.71°C/min) heats up 42.0% faster than Drive A (0.50°C/min), 34.0% faster than Drive B (0.53°C/min), and 18.3% faster than Drive D (0.60°C/min). Over the 28.13 minutes active travel, Drive C's temperature rose by +20.0°C (to 59.0°C), peaking higher than all other drives (Drives A/B peaked at 53.0°C, and Drive D peaked at 50.0°C).

# 4. Recommended Inspection Actions

• Brake Circuit C Inspection: Measure hydraulic pressure in release line C. Verify it reaches 25-30 bar. Lower pressure will lead to partial brake engagement (brake drag).
• Wheel Hub & Gearbox: Jack up Wheel C, override brake pressure, and rotate Wheel C by hand. Check for drag and inspect Gearbox C oil for metal wear particles.
• Zapi ACE4 Controller Auto-Tuning: Connect the console to run the motor parameter auto-characterization routine for the new motor to ensure optimal control efficiency.

# 5. Telemetry Visualizations

## 5.1 Try 3 Time-Series Telemetry Plot

## 5.2 Multi-Trial Comparative Bar Chart (Try 1 vs. Try 2 vs. Try 3)

## 5.3 transC Motor Temperature Comparison (Try 1 vs. Try 2 vs. Try 3)