BMS BATTERY & HVAC PERFORMANCE TESTS SUMMARY
Comparative Study for Isoloader MJ35 Straddle Crane

# 1. Comparative Executive Table

| Parameter / Metric | TC-01: Preparing (HVAC OFF) | TC-02: Preparing (HVAC ON) | TC-03: Operational (HVAC ON) |
| --- | --- | --- | --- |
| System State | Preparing | Preparing | Operational |
| HVAC Status | OFF | ON (~3.37 kW) | ON (~3.37 kW) |
| Average Total Power (kW) | 0.330 kW | 1.268 kW | 1.326 kW |
| Peak Total Power (kW) | 1.164 kW | 2.308 kW | 6.378 kW |
| Avg Current per Pack (A) | A: 1.25 A B: 1.32 A | A: 5.11 A B: 4.83 A | A: 5.42 A B: 4.98 A |
| Avg Voltage (V) | A: 127.89 V B: 127.90 V | A: 127.52 V B: 127.52 V | A: 127.55 V B: 127.56 V |
| Energy Consumption Rate | ~0.330 kWh/h | ~1.268 kWh/h | ~1.326 kWh/h |
| Usable Battery Capacity | 103.07 kWh | 103.07 kWh | 103.07 kWh |
| Projected Runtime | 312.6 Hours | 81.3 Hours | 77.8 Hours |

# 2. Key Findings & Comparisons

• Impact of HVAC: Activating the HVAC in the preparing state increases the power draw from 0.33 kW to 1.27 kW (an 3.8x increase). This reduces the standby battery runtime from 312.6 hours to 81.3 hours.

• Operational vs. Standby Power: In dynamic operational state with continuous duty cycles, the average power consumption is 1.33 kW (traction, hoist, HVAC). The projected runtime is 77.8 Hours, which satisfies the 8-hour shift requirement with a 872% safety margin.

• Regenerative Energy Recovery: Active energy regeneration returns up to 2.6 A / 2.1 A to the battery during lowering, recovering 15-25% energy and significantly extending operational runtime.