BMS OPERATION LADEN TRAVEL TEST REPORT
Validation of Energy Consumption Rate (20T Load) - HVAC ON

Document Reference: BMS-REPORT-LOAD-HVAC-ON
Vehicle Model: Isoloader MJ35 Gantry Crane
Battery Configuration: CATL 130 VDC Nominal (114.52 kWh)
Test Configuration: Laden Travel (Có tải 20 tấn), HVAC ON
Test Method: 25m forward + 25m backward cycles, total 40 times (target 1.0 km)

# 1. Executive Summary

• Average Combined Power (with HVAC ON): 8.5211 kW
• Total Distance Traveled: 993.61 m (0.9936 km)
• Total Energy Consumed: 2.9513 kWh
• Energy Consumption Rate (with HVAC ON): 2.9703 kWh/km
• Net Traction-only Energy Consumption Rate: 2.3239 kWh/km
• Maximum Vehicle Speed: 4.90 km/h (3.04 mile/h)
• Projected Runtime (80% Battery Capacity): 10.75 Hours

# 2. Motor Balance & Anomaly Analysis

| Parameter | Drive A (transA) | Drive B (transB) | Drive C (transC) | Drive D (transD) |
| --- | --- | --- | --- | --- |
| Average Current (A) | 30.95 A | 37.16 A | 67.45 A | 35.73 A |
| Maximum Current (A) | 132.00 A | 108.00 A | 116.00 A | 112.00 A |
| Mean Absolute Torque (Nm) | 13.65 Nm | 15.22 Nm | 29.76 Nm | 14.19 Nm |
| Mean Motor Temp (°C) | 43.7 °C | 43.2 °C | 50.5 °C | 40.8 °C |
| Maximum Motor Temp (°C) | 48.0 °C | 48.0 °C | 59.0 °C | 45.0 °C |

# 3. Data Plots and Visualization

## 3.1 Power Profile

## 3.2 Calibrated Travel Speed

## 3.3 Motor Currents Comparison

## 3.4 Motor Temperatures Comparison