Technical Specification

| Models by Stacking Height |  |
| --- | --- |
| ISO-35 | Lift 1x9’6” over 1x9’6” containers |
| ISO-35-3H | Lift 1x9’6” over 2x9’6” containers |

| Machine Specifications |  |
| --- | --- |
| Travel Speed Unload | Variable to 8km/h |
| Travel Speed Load | Variable to 5km/h |
| Lifting Capacity | 35 T |
| Lifting Speed Unload | 10 m/min |
| Lifting Speed Load | 6 m/min (Experimental 20T: 5.64 m/min Raise / 4.59 m/min Lower) |
| Lifting System | 4x Electric Motors with integrated electric brakes |
| Wheels | 4 |
| Steer Wheels | 4 Wheel steer driven by 4 electric motors, ±45° (up to ±90° in lateral mode) |
| Turn Radius | 10m (to outside wheel) |
| Drive Wheels | 4 Wheel drive |
| Grade | 5% standard (laden/unladen) |
| Spreader | Options: - 6m fixed top-lift spreader with auxiliary chains for 12m containers - 6m – 12m telescoping top-lift spreader |
| Suspension | Rigid mounting (no suspension) |
| Spreader Side Shift | ±250 mm (independent control at each end, allowing skew adjustment) |
| Braking | Regenerative dynamic braking via electric motors |
| Parking & Emergency Brake | Fail-safe spring-applied, electro-hydraulically released safety brake on motor shaft / gearbox input |
| Drive | AC Electric Motors coupled to Planetary Gear Hubs - 4 Wheel Drive |
| Tyres | Pneumatic, treaded |
| Battery pack | CATL, 130 VDC nominal (114.52 kWh total capacity, 2x 57.26 kWh packs in series; designed for minimum 8 hours of continuous operation on a single charge under typical duty cycles) |
| Charging Time (to 80% SOC) | Approx. 4.5 to 6 hours with a 15–20 kW charger (from 10% SOC to 80% SOC) |
| Energy Consumption | Traction: ~1.5 kWh/km (unladen) to ~3.5 kWh/km (laden); Hoist: ~0.15 kWh/cycle (20T lift/lower); Standby: ~3.7 kWh/h (with HVAC ON) |
| Regenerative Energy | 15-25% recovery (Traction/Travel), up to 45-50% recovery (Winch/Lowering 20T load) |
| Projected Winch Cycles (80% SOC) | Dự báo số chu kỳ nâng hạ với 80% dung lượng pin (91.62 kWh khả dụng) Trường hợp KHÔNG tính năng lượng thu hồi (Giới hạn tiêu hao thô): Xe thực hiện được 280 chu kỳ nâng và hạ liên tục. Trường hợp CÓ tính năng lượng thu hồi (Tiêu hao thuần thực tế): Xe thực hiện được 536 chu kỳ nâng và hạ liên tục. (Số chu kỳ thực tế sẽ dao động từ 280 đến 536 lượt tùy thuộc vào hiệu suất nạp/xả của pin và mức độ hao phí nhiệt lượng). |
| Warning Devices | 4x Flashing amber travel beacons, 1x Audible backup beeper, Horn |
| Machine Weight (6m-12m Spreader) | 17 Tons (approx. with 6m-12m telescoping spreader) |
| Max. Wheel Load | 18 Tons (or 180 kN) |
| Lights | Working lights, Driving lights, Side lights |
| Camera | Camera with Split Screen in Cab |

| Cab Specifications |  |
| --- | --- |
| Cab Position | Above side beam |
| Cab Type | Fully enclosed with sliding window, isolation mounted |
| Air Conditioning | Cooling/Heat |
| Control Console | Full operational control is from the cab |
| Operator indicators | Touchscreen display (HMI) for vehicle telemetry, including battery state of charge (SOC), system diagnostic codes, steering mode, and spreader status (twistlocks locked/unlocked, spreader position). |
| Steering control | Joystick Right (Y-axis) |
| Hoist control | Scroll button on Joystick Right |
| Travel control | Joystick Left (Y-axis): Push forward to travel Forward, pull backward to travel Reverse |
| Spreader control | Twistlocks: Buttons on Right joystick head (Bit 0: Lock, Bit 1: Unlock). Telescoping: Buttons on Right joystick head (Bit 2: Retract, Bit 3: Extend). Sideshift: Left joystick X-axis (Sideshift AB) and Right joystick X-axis (Sideshift CD) for independent control and skew adjustment. |
| Seat | Ergonomic adjustable seat with independent suspension |