Performance Test SOP - Executive Summary

45-Ton Battery-Powered Straddle Carrier

Document Date: December 17, 2025
Version: 1.0 - Initial Campaign (Measure & Learn)

OVERVIEW

This Standard Operating Procedure (SOP) defines the performance testing protocol for the 45-ton battery-electric straddle carrier. It is a Measure & Learn campaign designed to establish baseline performance data and validate all critical subsystems before formal acceptance criteria are defined.

Purpose

- Measure actual performance values under various operational conditions

- Establish baseline data for future reference and comparison

- Validate all subsystems (lift, sideshift, traction, thermal management, energy consumption)

- Develop Pass/Fail criteria based on measured data (after first campaign)

TEST CAMPAIGN SCOPE

8 Performance Tests

| Test ID | Test Name | Objective | Standard |
| --- | --- | --- | --- |
| PERF-001 | Lift Speed | Measure time to lift 45T from ground to 2H (max height) | ASME B30.2 |
| PERF-002 | Sideshift Speed | Measure horizontal movement speed at multiple height levels | AS2359 |
| PERF-003 | Acceleration | Validate traction motor performance; measure 0→50m in optimal time | CMAA adapted |
| PERF-004 | Regenerative Braking | Demonstrate regen braking reduces energy vs mechanical brake | Comparative analysis |
| PERF-005 | Turning Radius | Verify 4-wheel steer geometry; measure minimum turning radius | ISO 4310 |
| PERF-006 | Joystick Response | Assess operator control linearity, lag, dead zones | Qualitative |
| PERF-007 | Motor Thermal | Monitor motor/controller temps during continuous operation | Safety validation |
| PERF-008 | Energy per Cycle | Measure battery consumption per complete operational cycle | Baseline efficiency |

Test Conditions

- Load: 45-ton container (currently 20ft surrogate; actual weight TBD)

- Height Levels: Ground, Mid-height, 2H (max stacking)

- Repetition: 5 trials per test (except Turning Radius: 3 trials) → establishes Average ± StdDev

- Environmental Recording: Ambient temperature, humidity, floor condition, lighting

- Thermal Monitoring: Motor & controller temperatures tracked on every test (critical for battery vehicle)

KEY PRINCIPLES

- Measure First – Record actual values without pre-judging against theoretical specs

- Repeatability – 5 trials minimum establish statistical confidence (Average ± StdDev)

- Thermal Oversight – All tests include motor & controller temperature tracking

- Data Integrity – Technician signature & date on every sheet (legal documentation)

- Safety First – All tests conducted by certified technicians with accessible emergency stop

DELIVERABLES

Phase 1: Excel Data Collection

File: PERFORMANCE_TEST_DATA_FORM.xlsx

- 9 professional sheets: Cover + 8 test sheets (PERF-001 through PERF-008)

- Auto-calculating formulas: Average, Standard Deviation, unit conversions

- Thermal tracking: Motor & controller temperatures recorded per trial

- Signature fields: Technician & supervisor authorization

- Pass/Fail checkboxes: To be defined after first campaign

- Summary sections: Space for observations and anomalies

Phase 2: SOP Document

File: PERFORMANCE_TEST_SOP.docx

- 8+ detailed sections: Overview, Equipment, Pre-test Checklist, Detailed Procedures, Safety Protocols

- 30+ pages: Step-by-step guidance for each test

- Safety Protocols: Emergency procedures, thermal hazard management, battery safeguards

- Standards References: ASME B30.2, ISO 4310, AS2359, CMAA 70

EQUIPMENT & TOOLS REQUIRED

Vehicle Equipment

- HMI Display (SOC%, container position)

- Encoder Feedback (height/distance measurements)

- BMS Interface (SOC display on HMI)

Measurement Instruments

| Tool | Purpose | Accuracy | Qty |
| --- | --- | --- | --- |
| Stopwatch | Time measurement | 0.01 sec | 1 |
| Tape Measure | Distance measurements | ±50mm | 1 |
| IR Thermometer | Motor & controller temp | ±2°C | 1 |
| Chalk/Markers | Test zone marking | — | 1 set |
| Clipboard & Forms | Data recording | — | 1 |

Safety Equipment

- Hard hat & safety vest (required for all tests)

- High-visibility markers (perimeter marking)

- Emergency stop access (always available)

- First aid kit (on-site)

TEST EXECUTION WORKFLOW

Day-of-Test Preparation (30 min before start)

✓ Vehicle pre-check (battery ≥80%, temps normal, controls responsive)
✓ Test area marked (acceleration line, turning radius center, safety zone)
✓ Tools ready and verified (stopwatch, tape, IR thermometer, forms)
✓ Safety briefing completed (E-STOP location, hazard zones)
✓ Weather & floor condition noted in form
✓ Technician & supervisor contact info on file
✓ Ready to start PERF-001

Per-Test Execution

- Print form & verify legibility

- Verify measurement tools

- Record initial temperatures (motor, controller, ambient)

- Execute 5 trials with consistent technique

- Auto-calculate Average & StdDev in Excel

- Enter technician signature & date

- Complete summary comments

- Move to next test

Campaign Timeline

- Estimated Duration: 3–5 test days for all 8 tests

- Each Test: 30–45 minutes (setup + 5 trials + cool-down)

- Analysis: 1 day for data compilation and summary report

TEST HIGHLIGHTS

PERF-001: Lift Speed (ASME B30.2)

Establishes baseline lift performance in two joystick modes:

- Mode A: Full Joystick (100% max speed)

- Mode B: Normal Operation (~70% cruise speed)

Data: Time, distance, speed (m/s & km/h), motor/controller temps

PERF-004: Regenerative Braking (Comparative)

Compares energy efficiency:

- Series A: Regen Active (release joystick for automatic braking)

- Series B: Mechanical Brake (manual friction braking)

Result: Quantifies regen braking efficiency gain (target: 15–25% energy savings)

PERF-008: Energy per Cycle (Baseline Efficiency)

Measures real-world consumption:

- Profile A: HVAC OFF (baseline)

- Profile B: HVAC ON (realistic daily operation)

Data: 10 complete cycles per profile, SOC tracking, kWh calculation

Battery Reference: 114.52 kWh total capacity; 1% SOC = 1.1452 kWh

SAFETY PROTOCOLS

Critical Safeguards

- Emergency Stop: E-STOP tested before each test day; accessible at all times

- No Personnel Under Load: During lift tests, keep area clear

- Speed Limits: Acceleration ≤15 km/h; turning tests ≤5 km/h

- Thermal Limits: Stop if motor >85°C or controller >80°C; 15-min cool-down required

Battery Management

- SOC Floor: Never allow SOC to drop below 10% (prevents damage)

- Mid-Test Recharge: If SOC drops below 20%, pause and recharge to ≥80%

Incident Reporting

- Stop test immediately if anomaly detected

- Document: time, description, conditions

- Report to Project Engineer before resuming

FIRST CAMPAIGN DELIVERABLES

After completing all 8 tests, provide:

- Completed Data Excel – All 8 sheets with measured values, averages, std devs

- Summary Report – Executive summary, trends, thermal assessment, energy baseline

- Pass/Fail Recommendations – Engineering defines acceptance thresholds based on actual data

- Next Steps – Recommendations for future testing (e.g., full 45T load validation, extended endurance)

DATA RECORDING & CALCULATIONS

Excel Auto-Calculations

All formulas pre-built for accuracy:

Average = AVERAGE(Trial1:Trial5)
StdDev = STDEV(Trial1:Trial5)
Speed (km/h) = Speed (m/s) × 3.6
Energy (kWh) = Battery Capacity (kWh) × ΔSOC% / 100

Measurement Accuracy Standards

| Parameter | Precision | Example |
| --- | --- | --- |
| Time (stopwatch) | 0.1 sec | 72.3 sec |
| Distance (tape) | 1 cm | 8.05 m |
| Temperature (IR) | 1°C | 82°C |
| SOC (HMI) | 1% | 78% |
| Speed (calculated) | 0.1 m/s or 0.1 km/h | 1.1 m/s or 4.0 km/h |

Reporting: Average ± StdDev with one decimal place

KEY ADVANTAGES OF THIS APPROACH

Scientific Rigor: Multiple trials establish statistical confidence (Average ± StdDev)

Thermal Safety: Continuous monitoring prevents motor damage and validates cooling system

Energy Baseline: Real consumption data supports operational cost projections and range estimates

Regen Validation: Comparative testing proves regenerative braking effectiveness

Legal Documentation: Technician signatures on all sheets ensure accountability and traceability

Flexible Pass/Fail: Criteria defined after data collection, avoiding bias

No Special Instrumentation: Uses standard tools (stopwatch, tape measure, IR thermometer) — cost-effective

Operator-Friendly: Qualitative assessments (joystick feel) capture practical operator experience

CONTACT & APPROVAL

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared by | Project Engineer | __________________ | __________ |
| Reviewed by | Quality Lead | __________________ | __________ |
| Approved by | Project Manager | __________________ | __________ |

STANDARDS REFERENCED

- ASME B30.2 – Overhead and Gantry Cranes (Safety Standard for Cableways, Cranes, Derricks, Hoists, Hooks, Jacks, and Slings)

- ISO 4310 – Straddle Carriers – General Safety Requirements and Test Code

- ISO 13849-1 – Safety of Machinery – Safety-Related Parts of Control Systems

- AS2359 – Australian Standard for Straddle Carrier Performance

- CMAA 70 – Specification for Top Running Bridge and Gantry Type Multiple Lift Cranes

NEXT STEPS

- Share files with test team and obtain sign-off

- Schedule test dates (3–5 consecutive days recommended)

- Brief certified technicians on SOP and safety protocols

- Prepare test area per Section 3.2 requirements

- Execute tests following detailed procedures in Section 4

- Compile results into summary report

- Present findings to engineering for Pass/Fail criteria definition

- Plan Phase 2 (extended endurance testing, full 45-ton validation, etc.)

Document Version: 1.0 (December 17, 2025)
Campaign Type: Measure & Learn – Initial Baseline
Status: Ready for Execution

END OF EXECUTIVE SUMMARY