PERFORMANCE TEST SOP

45-Ton Battery-Powered Straddle Carrier

DOCUMENT INFORMATION

| Item | Details |
| --- | --- |
| Document Title | Performance Test Standard Operating Procedure |
| Equipment | 45-Ton Battery Electric Straddle Carrier |
| Date Created | December 17, 2025 |
| Version | 1.0 - Initial Campaign (Measure & Learn) |
| Test Approach | Record actual values + Pass/Fail TBD after first campaign |

SECTION 1: OVERVIEW & OBJECTIVES

1.1 Purpose

This SOP defines the performance testing protocol for the 45-ton battery-powered straddle carrier. The primary objectives are:

- Phase 1 (Current): Measure and record actual performance values under various conditions

- Baseline Establishment: Create baseline performance data for future reference

- System Validation: Verify all subsystems (lift, sideshift, traction, thermal, energy) operate within safe parameters

- Pass/Fail Criteria Development: After first campaign, engineering will define formal acceptance criteria

1.2 Test Scope

- 8 Performance Tests covering: Lift, Sideshift, Acceleration, Regenerative Braking, Turning Radius, Joystick Response, Motor Thermal, Energy Consumption

- Load Condition: Current 20ft container (actual weight TBD); target will validate with 45-ton load

- Height Levels: Ground, Mid, 2H stacking as applicable per test

- Environmental Recording: Temperature, humidity, floor condition, lighting

1.3 Key Principles

- Measure First: Record actual values; don't pre-judge against theoretical specs

- Repeatability: 5 trials per test (except Turning Radius: 3 trials) to establish average ± standard deviation

- Thermal Monitoring: All tests include motor & controller temperature tracking (critical for battery vehicle)

- Data Integrity: Sign & date every test; geotagged photos optional but recommended for anomalies

- Safety First: All tests performed by certified technicians; emergency stop accessible at all times

SECTION 2: EQUIPMENT & TOOLS REQUIRED

2.1 Test Vehicle Equipment

- HMI Display: For reading SOC%, container position

- Encoder Feedback: For height/distance measurements (if accessible for calibration)

- BMS Interface: SOC display on HMI

2.2 Measurement Instruments

| Instrument | Purpose | Accuracy | Quantity |
| --- | --- | --- | --- |
| Stopwatch | Time measurement (lift, sideshift, accel) | 0.01 sec | 1 |
| Tape Measure | Distance (acceleration 50m, sideshift stroke, turning radius) | ±50mm | 1 |
| IR Thermometer | Motor & controller temperature | ±2°C | 1 |
| Chalk/Markers | Mark test zones | - | 1 set |
| Clipboard & Forms | Data recording | - | 1 |

Table 1: Measurement Instruments Required for Testing

2.3 Safety Equipment

- Hard Hat & Safety Vest: Worn during all tests

- High-Visibility Markers: Mark test area perimeter

- Emergency Stop Access: Ensure E-STOP button accessible

- First Aid Kit: On-site availability

2.4 Test Environment

- Floor: Smooth cement, 0% slope, dry condition preferred

- Area: Clear 100m × 50m for acceleration & turning radius tests

- Weather: No active rain; temperature 10–35°C preferred

- Lighting: Adequate daylight or work lights; document condition

SECTION 3: PRE-TEST CHECKLIST (BEFORE EACH TEST DAY)

3.1 Vehicle Inspection

- [ ] Battery SOC ≥ 80% at start of test day

- [ ] Motor & Controller Temperatures at ambient (not elevated from previous operation)

- [ ] Hydraulic Oil Level within range; check for leaks (sideshift lines, hoist)

- [ ] Joystick & Controls responsive; no sticking or dead zones

- [ ] Spreader Locking Pins engaged when not testing

- [ ] Tires/Wheels condition & pressure check (if pneumatic); steering smooth

- [ ] Emergency Stop Button tested—cuts power immediately

- [ ] HMI Display powers on; SOC, position readings visible

- [ ] Encoder Calibration (if used)—verify zero reference

3.2 Test Area Setup

- [ ] Acceleration Track: Mark 0m and 50m points clearly with chalk/tape (straight, level surface)

- [ ] Turning Radius Zone: Clear 15m × 15m area for 360° circle

- [ ] Flat Surface Test Zone: Level surface, minimum 2m clearance on both sides

- [ ] Lift Test Zone: Overhead clearance ≥3m above 2H position; no obstructions

- [ ] Perimeter Safety: Mark test area with cones/barriers; post "Testing in Progress" sign

3.3 Measurement Tools Verification

- [ ] Stopwatch: Battery fresh, time synchronized with technician watch

- [ ] Tape Measure: Unfurl and verify no kinks; tension consistent

- [ ] IR Thermometer: Battery good, calibration recent (test on ice water ~0°C)

- [ ] Clipboard: Forms printed & legible; pen working

3.4 Documentation

- [ ] Test Date: _____________________

- [ ] Technician Name: _____________________

- [ ] Supervisor Name: _____________________

- [ ] Ambient Temperature: _________ °C

- [ ] Floor Condition: ☐ Dry ☐ Damp ☐ Wet | ☐ Smooth ☐ Rough

- [ ] Weather: ☐ Sunny ☐ Cloudy ☐ Overcast

- [ ] All pre-test items signed off: _____________________

SECTION 4: DETAILED TEST PROCEDURES

PERF-001: LIFT SPEED (45T → 2H)

Objective: Measure time to lift 45-ton load from ground to 2H (max height), establish baseline speed profile

Standard Reference: ASME B30.2 Crane Load Test

Condition: Container 20ft (actual weight TBD); both "Full Joystick" and "Normal Operation" modes

PERF-001 Setup

- Load Positioning:

  - Place 45-ton container (or 20ft surrogate) at ground level, centered under spreader

  - Verify container weight recorded in form

  - Ensure no obstacles above (trees, cables, lights) up to 10m height

- Height Reference:

  - Design 2H height (from DVP&R spec): measure from ground to bottom of container at maximum lift position using encoder readout OR

  - If encoder unavailable: use laser distance meter / measuring tape to establish baseline (first test only, then fixed for future tests)

  - Record in form: "Design 2H = _____ m"

- Temperature Baseline:

  - Measure motor & controller temps with IR thermometer before first trial

  - Record "Start Temp" in form

  - Allow ≥2 minutes between trials for thermal stabilization

- Joystick Modes:

  - Mode A - "Full Joystick": Push control lever to maximum (100% joystick travel) → maximum speed modulation

  - Mode B - "Normal Operation": Operator comfortable cruise speed (~70% joystick) → typical daily use speed

PERF-001 Procedure (5 Trials per Mode)

Trial Process:

FOR EACH TRIAL (A1–A5, then B1–B5):

- START MEASUREMENT:

  - Position container at ground level (spreader at minimum height)

  - Verify motor/controller temps at safe level (<50°C)

- START TIMER → Press "Lift Up" with selected joystick mode:

  - Mode A: Full joystick (max speed)

  - Mode B: Normal operation (~70% joystick)

- MONITOR LIFT:

  - Watch container rise smoothly

  - Record any hesitation, jerking, or speed variation

  - Observe motor/controller—no excessive noise or vibration

- STOP TIMER when container reaches 2H limit (HMI position max OR physical limit switch activates)

- HOLD at 2H: Leave container raised for 5 seconds

  - Verify spreader locks at height (no drifting)

  - Check stability (no tilt/sway)

- LOWER container back to ground (any speed—not part of this test)

- COOL DOWN: Wait 2–3 minutes before next trial

  - Measure motor/controller temps

  - If motor temp >60°C, wait longer

- RECORD in Excel form:

  - Travel Distance (m): from encoder or tape measure (typical ~8–8.5m)

  - Time (s): timer reading

  - Speed (m/s): calculated as Distance/Time

  - Speed (km/h): calculated as Speed(m/s) × 3.6

  - Motor/Controller temperatures (start & end)

Excel Auto-Calc:

- Average of 5 trials → Compare Mode A vs Mode B

- StdDev shows consistency

Acceptance Criteria:

- TBD (Record actual times; typical industrial crane: 60–90 sec for 8m lift)

- Flag if >120s or <40s for review

PERF-002: SIDESHIFT SPEED (0→FULL STROKE)

Objective: Measure sideshift mechanism speed across height levels; validate smooth horizontal movement

Standard Reference: AS2359 (Australian Straddle Carrier Standard)

Condition: 45-ton load at Ground / Mid / 2H heights; Full Joystick + Normal modes

PERF-002 Setup

- Measure Full Stroke:

  - Lower container to ground, spreader to center position

  - Mark center position with chalk

  - Engage full left sideshift joystick → measure distance to left limit (e.g., 300mm)

  - Return to center, engage full right → measure to right limit

  - Total Stroke = Left distance + Right distance (typical 500–700mm)

  - Record in form: "Full Stroke = _____ mm"

- Test Heights:

  - Test 1: Container at ground level

  - Test 2: Container at mid-height (~0.5×2H)

  - Test 3: Container at 2H (max height)

  - Repeat all three heights for each mode

- Joystick Modes:

  - Same as PERF-001 (Full vs Normal)

PERF-002 Procedure (5 Trials per Height Level)

FOR EACH HEIGHT (Ground → Mid → 2H):

- POSITION CONTAINER at target height, spreader centered

- MEASURE BASELINE:

  - IR temp check (motor, controller)

  - Note hydraulic pressure gauge if visible (record only, do NOT interpret)

- START TIMER → Engage sideshift joystick (full travel to one side):

  - Mode A: Full joystick (max speed)

  - Mode B: Normal (~70%)

- STOP TIMER when spreader reaches limit stop (hydraulic jack fully extended)

- RECORD:

  - Stroke traveled (mm)—pre-measured full stroke value

  - Time (s)

  - Speed (mm/s) = Stroke / Time

  - Motor/Controller temps

- RETURN to center (any speed)

- COOL DOWN: 2–3 min

- REPEAT opposite direction (left → right OR right → left)

- REPEAT 5 TIMES total for this height level

Acceptance Criteria:

- TBD (Typical straddle: 20–30 sec for 600mm; record actual, hydraulic pressure dependent on load)

- Flag if >45s or jerky movement

PERF-003: ACCELERATION (0→15KM/H, 50M)

Objective: Measure traction motor performance; validate smooth acceleration profile; measure coast-down distance

Standard Reference: CMAA Overhead Crane Performance Test (adapted for ground-level mobile equipment)

Condition: Empty + Loaded 45-ton

PERF-003 Setup

- Mark Test Track:

  - Choose straight, level cement floor area

  - Mark 0m (start line) and 50m (finish line) with chalk

  - Mark intermediate points: 10m, 20m, 30m, 40m (helps observe acceleration profile)

  - Ensure no obstacles in lane; clear for coast-down extension (+100m)

- Initial Positioning:

  - Empty run: Vehicle at 0m, ready to accelerate

  - Loaded run: Container 20ft at 2H height (full lift), then move backward to 0m start point, ready to drive forward

- Thermal Check:

  - Both motor & controller temps at ambient

  - Battery SOC ≥ 60% before starting

PERF-003 Procedure (5 Trials Empty, 5 Trials Loaded)

ACCELERATION PHASE:

- Position vehicle at 0m start line

- START TIMER → Full throttle joystick:

  - Accelerate toward 50m mark as quickly as possible

  - Maintain straight line (no steering inputs)

- RECORD TIME when front of vehicle reaches 50m finish line

  - Calculate: Acceleration Speed = 50m / Time(sec) → convert to km/h

  - Typical target: <10s (empty), <15s (loaded)

- COAST-DOWN PHASE (immediately after reaching 50m):

  - Release joystick fully (no throttle input)

  - Vehicle will decelerate via regenerative braking + friction

  - START NEW TIMER at the 50m point (vehicle still moving)

  - RECORD DISTANCE when vehicle reaches ~5km/h (or nearly stops)

  - Calculate coast distance: _________ m

  - Coast Time: _________ sec

  - This proves regen braking is active (compare to mechanical braking only as baseline)

- THERMOMETER CHECK:

  - Measure motor temp immediately after coast stop

  - Record Peak Temp (should be <70°C for continuous use)

- COOL DOWN: Return to start (any speed), wait 3–5 min

  - Measure temps again (should drop)

  - If motor temp >75°C, extend cool time

- REPEAT 5 times for Empty condition, then repeat 5 times for Loaded condition

Acceptance Criteria:

- TBD (Design target: Empty <10s, Loaded <15s for 50m; confirm after first campaign)

- Monitor coast distance; longer coast = better regen efficiency proof

PERF-004: REGENERATIVE BRAKING (COMPARATIVE)

Objective: Demonstrate regen braking reduces energy consumption vs. mechanical brake; measure SOC efficiency

Approach: Comparative testing (Regen profile vs. Mechanical Brake profile) rather than absolute efficiency % (no current sensor available)

PERF-004 Setup

- Define Test Cycle:

  - 10 consecutive repetitions of: Accelerate 30 sec (full throttle) → Coast 10 sec (release joystick, regen active)

  - Distance approximate: ~200–300m per cycle

- SOC Measurement:

  - Read SOC% from HMI before test series

  - Read SOC% after 10 cycles

  - Calculate: ΔSOC = SOC_start – SOC_after_10_cycles

- Thermal Monitoring:

  - Track motor & controller temps throughout (record peak)

PERF-004 Procedure

SERIES A: REGEN ACTIVE (Baseline)

- SOC_start = _____ %

- FOR 10 CYCLES:

  - Accel 30 sec: full throttle joystick

  - Coast 10 sec: release joystick completely (regen braking engages automatically)

  - No manual brake input

  - Smooth, consistent driving

- SOC_after_10_cycles_regen = _____ %

- ΔSOC_Regen = SOC_start – SOC_end = _____ %

- kWh used (calc) = 114.52 kWh (total battery) × ΔSOC% / 100 = _____ kWh

- Motor Peak Temp (°C): _____

SERIES B: MECHANICAL BRAKE PROFILE (Control)

- Allow 30 min cool-down (return to ambient temp)

- SOC_start_mechanical = _____ %

- FOR 10 CYCLES (same track):

  - Accel 30 sec: full throttle joystick

  - BRAKE 10 sec: engage manual brake pedal (minimize regen, use friction brake)

  - More aggressive stopping (typical baseline)

- SOC_after_10_cycles_mechanical = _____ %

- ΔSOC_Mechanical = SOC_start_mechanical – SOC_after_10_cycles_mechanical = _____ %

- kWh used (calc) = 114.52 × ΔSOC_mechanical% / 100 = _____ kWh

- Compare:

  - kWh_Regen vs kWh_Mechanical

  - Improvement% = (1 – kWh_Regen / kWh_Mechanical) × 100% = _____ %

  - Target: Regen should consume ~15–25% LESS energy than mechanical brake

Acceptance Criteria:

- Regen Proven if: Smooth deceleration + ΔSOC_Regen < ΔSOC_Mechanical + No harsh braking vibration

- Flag if mechanical brake needed to stop (indicates potential regen loss)

PERF-005: TURNING RADIUS (4WS, ISO 4310)

Objective: Verify 4-wheel steer system geometry; measure minimum turning radius at full load

Standard Reference: ISO 4310 (Straddle Carriers Performance Test)

Load: 45-ton container at Ground & 2H heights

PERF-005 Setup

- Prepare Test Area:

  - Flat, open area (parking lot / warehouse floor) ~30m × 30m

  - Mark center point with chalk ⊙ (approx. pivot point during turn)

  - Pre-draw large circle (radius ~6m) around center point using chalk line or rope

- Height Test Conditions:

  - Test 1: Container at ground level (low center of mass, better stability)

  - Test 2: Container at 2H height (worst-case: high COG, more tilt during turn)

- Speed Control:

  - Low speed turn: 3–5 km/h (per ISO standard)

  - Engage 4-wheel steer mode (if button/switch available on HMI)

PERF-005 Procedure (3 Trials per Height)

- POSITION vehicle 10m away from center ⊙, facing inward

- ENGAGE 4-WHEEL STEER MODE (full lock steering)

- START MOTION at 3–5 km/h toward center point

- SLOWLY TURN 360° around center ⊙:

  - Maintain low speed (walk speed)

  - Full steering lock applied (both front & rear wheels turned maximum)

  - Drive around center in full circle (2–3 minutes per circle)

- MEASURE INNER RADIUS:

  - After completing one 360° circle, place chalk marks at inner edge of tire path

  - Measure from center ⊙ to inner tire mark using tape measure

  - Record: Inner Radius = _____ m

- MEASURE OUTER RADIUS (optional but useful):

  - Measure from center ⊙ to outer tire mark

  - Outer Radius = _____ m

- RECORD OBSERVATIONS:

  - Steering smooth? (Y/N)

  - Container stable / no tilt warning? (Y/N)

  - Motor sound/temp normal? (Y/N)

  - Any slipping detected? (Y/N)

- REPEAT for same height (3 trials total) → Average radius

- REPEAT procedure for 2H height level

Acceptance Criteria:

- Design Target: ≤6m inner radius (per ISO straddle standard; validate after first campaign)

- Observation: 2H height should have slightly larger radius due to center-of-mass effect

- Flag if radius >7m or steering feels sluggish

PERF-006: JOYSTICK RESPONSIVENESS (QUALITATIVE)

Objective: Verify joystick control linearity & operator feel; detect lag, dead zones, or jerk

Approach: Qualitative assessment (operator feedback) since no CAN logger available

PERF-006 Setup

- Joystick Range:

  - Joystick neutral: 0% (no input)

  - Joystick full: 100% (maximum deflection)

  - Test points: 25%, 50%, 75%, 100%

- Test Motion: Smooth acceleration (not emergency stop)

- Operator: Certified technician familiar with vehicle

PERF-006 Procedure

FOR EACH JOYSTICK POSITION (25%, 50%, 75%, 100%):

- START from standstill

- APPLY JOYSTICK to target position (e.g., 50% = halfway push)

  - Hold steady for 3–5 seconds

- OBSERVE vehicle response:

  - Immediate response? (YES / NO)

  - Smooth acceleration curve? (YES / NO)

  - Any lag or delay? (YES / NO)

  - Any jerking / hesitation? (YES / NO)

- RECORD qualitative rating:

  - Speed Response: ☐ Smooth ☐ Slight Lag ☐ Noticeable Delay

  - Acceleration: ☐ Smooth ☐ Minor Jerk ☐ Significant Jerk

  - Linearity: ☐ Proportional (speed ∝ joystick) ☐ Non-linear (needs review)

- RELEASE joystick → vehicle coasts naturally

- NOTE any other observations (dead band, stickiness, creep, etc.)

- REPEAT for all joystick positions

Acceptance Criteria:

- PASS: No perceivable lag + smooth acceleration + proportional response

- FLAG: Any jerking, dead band, or delayed response → report to PLC calibration engineer

PERF-007: MOTOR THERMAL & CURRENT BEHAVIOR

Objective: Monitor motor & controller temperatures during continuous operation; detect thermal runaway or over-current

Condition: No Load + Full Load 45-ton (compare thermal signatures)

PERF-007 Setup

- Thermal Reference:

  - Ambient temperature baseline: _____ °C

  - Motor/Controller at ambient (idle ≥30 min)

- Continuous Operation:

  - Each test: 5–10 minutes of steady-state acceleration/braking (simulating warehouse cycle)

- Monitoring Interval: Temperature check every 1–2 minutes during test

PERF-007 Procedure

NO LOAD TEST:

- Vehicle empty, no container

- START OPERATION CYCLE:

  - Continuous acceleration (full throttle) for 5 min

  - Record motor temp: Start _____, 1min _____, 2min _____, ..., 5min (Peak) _____

  - Record controller temp: same interval

- BRAKE / COAST for 2 min (allow partial cooling)

- REPEAT cycle 2–3 times

- OBSERVATIONS:

  - Peak motor temp reached: _____ °C (target <70°C for continuous use)

  - Peak controller temp: _____ °C (target <60–65°C)

  - Any thermal runaway (temp keeps rising)? (Y/N)

  - Any overcurrent indicator or warning? (Y/N)

FULL LOAD TEST (45-ton at 2H):

- Container 45t at 2H height

- DRIVE continuous cycle:

  - Traction motion: full throttle x 5 min (move forward/backward if space allows)

  - Lift/lower cycles x 5 min (if drive motor separate from hoist motor)

  - Braking phase x 2 min

- Temperature trend (should be higher than no-load):

  - Motor Start: _____ °C

  - Motor Peak: _____ °C (expected <75–80°C under load)

  - Controller Start: _____ °C

  - Controller Peak: _____ °C (expected <65–70°C)

- NOTE thermal delta: Full Load Motor Temp – No Load Motor Temp = _____ °C delta

  - Typical: +10–20°C additional under load

Acceptance Criteria:

- PASS: Motor <80°C, Controller <70°C during 5-min continuous cycle

- CAUTION: If temp exceeds limits, reduce load/cycle duration and flag thermal management review

- FAIL: Thermal runaway (continuous rise despite reducing input) → investigate motor/cooling system

PERF-008: ENERGY PER CYCLE (LIFT+TRAVEL+LOWER)

Objective: Measure battery energy consumption per complete operational cycle; establish baseline for daily operational cost

Cycle Definition:

- Lift container from ground to 2H (full height)

- Travel horizontally 50m (forward or in warehouse pattern)

- Lower container back to ground (full descent)

HVAC Profile: Test both OFF (baseline) and ON (realistic daily operation)

PERF-008 Setup

- Battery Capacity Reference:

  - Total: 114.52 kWh (2 × 57.26 kWh packs in series)

  - Conversion: 1% SOC = 1.1452 kWh

- HMI SOC Display:

  - Ensure BMS-to-PLC communication stable

  - Allow ±1% SOC drift tolerance (measurement uncertainty)

- Test Cycles:

  - 10 complete cycles per profile (HVAC OFF, then HVAC ON)

  - Allow 5-minute rest between profiles for thermal stabilization

- Load: 45-ton container (or 20ft current load with actual weight recorded)

PERF-008 Procedure

PROFILE A: HVAC OFF (Baseline)

- PRE-TEST:

  - Ambient temp: _____ °C

  - Battery SOC before test: _____ %

  - Record: SOC_start_off = _____ %

- DISABLE HVAC (via HMI or manual switch)

- FOR 10 COMPLETE CYCLES:
Cycle #1:

  - Lift container (ground → 2H): ~ 1.5 min

  - Travel 50m: ~ 2 min (3–5 km/h speed)

  - Lower to ground: ~ 1.5 min

  - Total cycle time: ~5 min

- Repeat cycles 2–10 (each ~5 min)
Total duration: ~50 min

- POST-TEST:

  - Record: SOC_after_off = _____ %

  - ΔSOC_off = SOC_start_off – SOC_after_off = _____ %

  - Energy consumed = 114.52 kWh × ΔSOC_off% / 100 = _____ kWh

  - Energy per cycle = Total energy / 10 cycles = _____ kWh/cycle

- COOL DOWN: 5–10 min (allow motor/controller to cool)

PROFILE B: HVAC ON (Realistic Daily Operation)

- PRE-TEST:

  - Ambient temp: _____ °C

  - Battery SOC before test: _____ %

  - Record: SOC_start_hvac = _____ %

- ENABLE HVAC (cabin heating/cooling)

  - Set comfortable temperature (e.g., 22°C)

- FOR 10 COMPLETE CYCLES (same as Profile A):

  - Same Lift + Travel + Lower pattern

  - Duration: ~50 min total

- POST-TEST:

  - Record: SOC_after_hvac = _____ %

  - ΔSOC_hvac = SOC_start_hvac – SOC_after_hvac = _____ %

  - Energy consumed = 114.52 × ΔSOC_hvac% / 100 = _____ kWh

  - Energy per cycle = Total energy / 10 cycles = _____ kWh/cycle

- HVAC PENALTY:

  - Additional energy vs HVAC OFF = (Energy_hvac – Energy_off) / Energy_off × 100% = _____ %

Acceptance Criteria:

- TBD (Record actual; typical battery vehicle: 0.3–0.6 kWh/cycle; define target after first campaign)

- HVAC Impact: Expect 10–20% additional consumption (normal for HVAC load)

- Flag if energy per cycle >0.8 kWh → investigate thermal/mechanical inefficiency

SECTION 5: DATA RECORDING & CALCULATIONS

5.1 Excel Form Structure

Each Test Sheet (PERF-001 to PERF-008) Contains:

| Section | Content |
| --- | --- |
| Header | Test Name, Date, Technician, Ambient Temp, Condition Notes |
| Trial Table | 5 rows (Trial 1–5), measurement columns, auto-calc Average & StdDev |
| Record Column | Space for technician to enter actual observed value (qualitative) |
| Pass/Fail Column | Checkbox PASS / FAIL (editable; to be defined after first campaign) |
| Signature | Technician signature & date (legal documentation) |
| Summary | Free text: key findings, anomalies, recommendations |

Table 2: Excel Test Sheet Structure

5.2 Auto-Calculated Formulas

Average (PERF-001 example):
=AVERAGE(B6:F6)

Standard Deviation:
=STDEV(B6:F6)

Converted Units (e.g., m/s to km/h):
=Speed_m_s × 3.6

Energy Calculation (PERF-008 example):
=114.52 × (SOC_start% – SOC_end%) / 100

5.3 Measurement Accuracy & Significant Figures

| Parameter | Precision | Example |
| --- | --- | --- |
| Time (stopwatch) | 0.1 sec | 72.3 sec |
| Distance (tape) | 1 cm | 8.05 m |
| Temperature (IR) | 1°C | 82°C |
| SOC (HMI) | 1% | 78% |
| Speed (calculated) | 0.1 m/s or 0.1 km/h | 1.1 m/s or 4.0 km/h |

Table 3: Measurement Accuracy Requirements

Rounding Rule: Report average ± std dev with one decimal place

SECTION 6: SAFETY PROTOCOLS

6.1 During Testing

- Emergency Stop: E-STOP button tested before each test day; accessible at all times

- No Personnel Under Load: During lift tests, no one stands beneath or near container

- Clearance Check: Before acceleration/turning tests, clear test area of personnel and obstacles

- Speed Limits: Acceleration tests limited to 15 km/h; steering tests to 5 km/h

- Communication: If multi-technician, use clear verbal signals or radio comms

6.2 High-Temperature Hazard

- Motor Thermal: If motor exceeds 85°C, stop test immediately and cool for 15 min

- Controller Thermal: If controller exceeds 80°C, cease operation

- Cool-Down Protocol: Between trials, measure temp; if >60°C, wait 3–5 min before next trial

6.3 Battery Management

- SOC Floor: Do not allow battery SOC to drop below 10% during testing (prevent damage)

- Mid-Test Recharge: If SOC drops below 20% mid-campaign, pause testing and recharge to ≥80%

6.4 Incident Reporting

If any anomaly detected (noise, vibration, overheat, loss of control, etc.):

- Stop test immediately

- Document: time, description, conditions

- Report to Project Engineer before resuming

SECTION 7: FIRST CAMPAIGN DELIVERABLES

After completing all 8 tests (estimated 3–5 test days), provide:

- Completed Data Excel: All 8 sheets filled with measured values, averages, std devs

- Summary Report:

  - Executive summary of each test result

  - Trends & observations

  - Thermal performance assessment

  - Energy baseline

- Pass/Fail Recommendation: Engineering to define acceptance thresholds based on measured data

- Next Steps: Recommendations for future testing (e.g., full 45-ton load validation, extended endurance test)

SECTION 8: REFERENCES & STANDARDS

- ASME B30.2: Overhead and Gantry Cranes (Safety Standard for Cableways, Cranes, Derricks, Hoists, Hooks, Jacks, and Slings)

- ISO 4310: Straddle Carriers – General Safety Requirements and Test Code

- ISO 13849-1: Safety of Machinery – Safety-Related Parts of Control Systems

- AS2359: Australian Standard for Straddle Carrier Performance

- CMAA 70: Specification for Top Running Bridge and Gantry Type Multiple Lift Cranes

DOCUMENT APPROVAL

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared by | Project Engineer | __________________ | __________ |
| Reviewed by | Quality Lead | __________________ | __________ |
| Approved by | Project Manager | __________________ | __________ |

Table 4: Document Approval Signatures

APPENDIX A: QUICK REFERENCE CHECKLIST

Day-of-Test Checklist (30 min before start):

- Vehicle pre-check (battery >80%, temps normal, controls responsive)

- Test area marked (acceleration line, turning radius center, safety zone)

- Tools ready (stopwatch, tape, IR thermometer, forms, clipboard)

- Safety briefing completed (E-STOP location, hazard zones)

- Weather / floor condition noted in form

- Technician & supervisor contact info on file

- Ready to start PERF-001

Per-Test Checklist:

- Form printed & legible

- Measurement tools verified

- Initial temperatures recorded (motor, controller, ambient)

- 5 trials executed with consistent technique

- Average & StdDev calculated in Excel

- Signature & date entered

- Summary comments completed

- Move to next test

Version History:

- v1.0 (17 Dec 2025): Initial SOP – Measure & Learn Campaign for 45T Battery Straddle Carrier

END OF PERFORMANCE TEST SOP