Technical Specification

| Models by Stacking Height |  |
| --- | --- |
| ISO-35 | Lift 1x9’6” over 1x9’6” containers |
| ISO-35-3H | Lift 1x9’6” over 2x9’6” containers |

| Machine Specifications |  |
| --- | --- |
| Travel Speed | Variable to 9km/h ?? |
| Travel Speed Unload | Variable to 8km/h |
| Travel Speed Load | Variable to 6km/h |
| Lifting Capacity | 35 T |
| Lifting Speed Unload | 10 m/min |
| Lifting Speed Load | 6 m/min |
| Lifting System | Electric, 4x Electrical Motors |
| Wheels | 4 |
| Steer Wheels | 4 Wheel steer, 90deg Wheel turn |
| Turn Radius | 10m (to outside wheel) |
| Drive Wheels | 4 Wheel drive |
| Grade | 5% (Option, Higher Grades) |
| Spreader | Options: - 6m fixed top-lift spreader with auxiliary chains for 12m containers - 6m – 12m telescoping top-lift spreader |
| Suspension | Option for rough ground or poor pavement |
| Spreader Side Shift | +/-125mm (Independent at each end) |
| Braking | Hydrostatic (dynamic) braking |
| Parking Brake | Brake on the drive wheels |
| Drive | Gear Hubs coupled to hydraulic motors |
| Tyres | Pneumatic, treaded |
| Battery pack | CATL, XXX Volts (xx working hours) |
| Energy Consumption |  |
| Regenerative Energy | 70 % (from braking and lowering spreader) |
| Warning Devices | Options: 2x Flash amber travel beacons 1x Audible beeper, Horn |
| Machine Weight (6m-12m Spreader) | 17T ? |
| Max. Wheel Load | ? T |
| Lights | Working lights, Driving lights, Side lights |
| Camera | Camera with Split Screen in Cab |

| Cab Specifications |  |
| --- | --- |
| Cab Position | Above side beam or front mounted |
| Cab Type | Fully enclosed with sliding window, isolation mounted |
| Air Conditioning | Cooling/Heat |
| Control Console | Full operational control is from the cab Option: Additional radio controls for pedestrian operators. |
| Operator indicators |  |
| Steering control | Joystick Left |
| Hoist control | Scroll button on Joystick Left |
| Spreader control |  |
| Seat | Ergonomic adjustable seat with independent suspension |