MULTI-TRIAL TELEMETRY REPORT: TRAVEL SYSTEM THERMAL PERFORMANCE

Evaluation of laden heating and natural cooling telemetry across 5 trials

Vehicle Model: Isoloader MJ35 Gantry Crane
Test Load: 20 Tons (Laden Travel)
Analysis Date: June 29, 2026
Document Status: Final Report

1. Executive Summary & Test Overview

This report compiles and analyzes the motor temperatures and operational currents of the travel system (transA, B, C, D) on the Isoloader MJ35 gantry crane across 5 trials. The testing program evaluated both laden travel runs under a 20-ton load and natural cooling phases when the vehicle was stationary. The data is extracted directly from the system's PLC telemetry logs.

The primary goals are to evaluate thermal stability under laden conditions, verify HMI temperature warning indications, test PLC thermal cutback limits, and analyze the cooling rates. Additionally, a persistent abnormal thermal and electrical signature was detected on Travel Drive C (transC), which requires motor replacement to rule out any vehicle architectural influences.

2. Laden Travel Telemetry Comparison (Heating Phase)

The table below summarizes the thermal development and average current draws during active travel under a 20T load. Trial 4 is classified as Aborted due to its short duration (4.03 min) and lack of current/heating data. Trial 5 represents the longest continuous travel run of 45.51 minutes (approx. 1.5 - 2.0 km).

| Motor ID | Try 1 (21.5 min) | Try 2 (22.3 min) | Try 3 (34.6 min) | Try 5 (45.5 min) | Multi-Trial Avg |
| --- | --- | --- | --- | --- | --- |
| transA | 39°C -> 48°C +9.0°C ¦ 27.3A | 45°C -> 54°C +9.0°C ¦ 27.8A | 44°C -> 54°C +9.0°C ¦ 25.8A | 54°C -> 68°C +14.0°C ¦ 27.6A | +10.3°C 27.1A |
| transB | 38°C -> 48°C +10.0°C ¦ 29.3A | 44°C -> 56°C +10.0°C ¦ 33.3A | 42°C -> 56°C +12.0°C ¦ 30.5A | 54°C -> 71°C +17.0°C ¦ 31.6A | +12.3°C 31.2A |
| transC | 40°C -> 59°C +17.0°C ¦ 48.6A | 48°C -> 64°C +16.0°C ¦ 45.3A | 45°C -> 64°C +17.0°C ¦ 42.2A | 59°C -> 85°C +26.0°C ¦ 46.6A | +19.0°C 45.7A |
| transD | 36°C -> 45°C +9.0°C ¦ 28.9A | 42°C -> 51°C +9.0°C ¦ 31.2A | 40°C -> 53°C +11.0°C ¦ 30.8A | 51°C -> 67°C +14.0°C ¦ 29.0A | +10.8°C 30.0A |

Travel Drive C Anomaly: Drive C (transC) consistently draws 50-70% more average current compared to its symmetrical counterparts (45.7A avg vs. ~29.4A). Consequently, its temperature rise is almost double, peaking at 85.0°C in Trial 5. This triggers the 85.0°C system temperature warning limit.

3. Natural Cooldown Telemetry Comparison (Cooling Phase)

The table below details the temperature decay during stationary parked periods. Note on Try 5: Due to a 9.20-minute unlogged gap at the start of cooldown, the actual temperature drop for Drive C over the entire 25.57-minute rest period was from 85.0°C to 71.0°C (a drop of 14.0°C at 0.55°C/min).

| Motor ID | Try 1 (59.9 min) | Try 2 (10.7 min) | Try 3 (59.4 min) | Try 5 (16.4 min)* |
| --- | --- | --- | --- | --- |
| transA | 47°C -> 45°C -2.0°C ¦ -0.03°C/m | 57°C -> 57°C 0.0°C ¦ 0.00°C/m | 56°C -> 53°C -3.0°C ¦ -0.05°C/m | 64°C -> 64°C 0.0°C ¦ 0.00°C/m |
| transB | 47°C -> 44°C -3.0°C ¦ -0.05°C/m | 57°C -> 56°C -1.0°C ¦ -0.09°C/m | 56°C -> 53°C -3.0°C ¦ -0.05°C/m | 65°C -> 64°C -1.0°C ¦ -0.06°C/m |
| transC | 57°C -> 48°C -9.0°C ¦ -0.15°C/m | 60°C -> 60°C 0.0°C ¦ 0.00°C/m | 59°C -> 54°C -5.0°C ¦ -0.08°C/m | 76°C -> 71°C -5.0°C ¦ -0.31°C/m |
| transD | 45°C -> 42°C -3.0°C ¦ -0.05°C/m | 56°C -> 54°C -2.0°C ¦ -0.19°C/m | 54°C -> 51°C -3.0°C ¦ -0.05°C/m | 62°C -> 60°C -2.0°C ¦ -0.12°C/m |

Newton's Law of Cooling: Cooldown rates are highly dependent on the temperature gradient. At 85°C (Try 5), Drive C cooled at 0.98°C/min initially. As the temperature fell to 76°C, the rate slowed to 0.31°C/min. A 20-minute pause is sufficient to reset the motor's thermal budget.

4. Engineering Evaluation & Conclusions

1. Drive C (transC) Abnormal Behavior: Drive C displays a persistent current draw discrepancy (+50% to +70% compared to Drive A, B, and D) and double the heating rate across all valid trials. This is an abnormal operational signature. To isolate and rule out any systemic effects from the crane's vehicle architecture, physical replacement is required.

2. PLC Protection & Safety Margins: The peak temperature of 85.0°C remains safely below the PLC's automatic thermal cutback threshold of 110.0°C. This confirms that the crane operates at full performance during all trials without premature software limitations. The HMI warning limit of 85.0°C functioned correctly to alert operators.

3. Duty Cycle Validation: Thermal telemetry indicates that a rest period of 20-25 minutes is adequate to lower temperatures from the 85°C warning limit to a stable baseline (~71°C), providing clear guidelines for continuous testing.

5. Actionable Maintenance Recommendations

Replace the Travel Drive C Motor Assembly: Abort further high-temperature travel runs and replace the transC motor assembly. This will isolate the problem and confirm if the discrepancy is driven by vehicle architecture or physical drive components.

Inspect Gearbox and Mechanical Hub C: Check oil levels, planetary gears, and bearings on Wheel C to check for mechanical friction.

Implement Temporary Runtime Limits: Until the motor is replaced, limit continuous laden travel to 30 minutes, followed by a 20-minute parked cooldown period.

Winch B Temperature Sensor Overhaul: Replace the faulted Winch B PT100 sensor (currently reading 0.0°C) to restore safety protections on the hoist system.