WINCH MOTOR THERMAL PERFORMANCE REPORT (TRIAL 2)

Document Reference: Winch-MJ35-THERMAL-TRY2
Test Date: June 24, 2026
Load: 20 Tons
Total Cycles: 15 Cycles

# 1. Executive Summary

This report provides a formal thermal evaluation of the Isoloader MJ35 winch system during its second trial (Try 2). The trial consisted of 15 complete raise and lower cycles with a 20T load. Telemetry logs indicate that Winch C is the hottest motor, reaching the targeted safety limit of 80.0°C at the end of Cycle 13. A 15-minute rest/cooling period was implemented, successfully reducing the motor temperature of Winch C by 15.0°C and allowing the operator to safely complete the remaining cycles.

# 2. Telemetry Temperature Metrics

| Winch Motor | Init Temp | Pre-Rest Temp | Post-Rest Temp | Final Temp | Peak Temp | Cooling Rate |
| --- | --- | --- | --- | --- | --- | --- |
| Winch A | 44.0°C | 76.0°C | 62.0°C | 70.0°C | 79.0°C | 0.93°C/min |
| Winch B | 0.0°C | 0.0°C | 0.0°C | 0.0°C | 0.0°C | N/A (Fault) |
| Winch C | 45.0°C | 80.0°C | 65.0°C | 73.0°C | 80.0°C | 1.00°C/min |
| Winch D | 42.0°C | 68.0°C | 59.0°C | 62.0°C | 68.0°C | 0.60°C/min |

# 3. Evaluation of 15-Minute Rest Period (Cycle 13)

The 15-minute rest period began at 11:41:27 and ended at 11:56:30. During this period, the motor temperatures dropped according to convective natural cooling. Winch C cooled down from 80.0°C to 65.0°C (a cooling rate of 1.00°C/min). Winch A cooled down by 14.0°C (cooling rate of 0.93°C/min), and Winch D cooled down by 9.0°C (cooling rate of 0.60°C/min). The cooling rates are proportional to the initial temperature, confirming standard convective behavior.

# 4. Active Heating & 80.0°C Limit Analysis

Winch C reached exactly 80.0°C at the end of the 13th cycle (stroke 26). The average heating rate during the active phase 1 was 1.09°C/min. After the rest period, the heating rate increased to 1.80°C/min. This shows that the motor was operating under high thermal stress. If the rest period was not implemented, Winch C would have reached the 85.0°C warning limit within 11 minutes (approx. 4 cycles). The 15-minute rest is therefore validated as an essential safety protocol.

# 5. Comparison with Trial 1 (First 5 Cycles)

A comparison of the first 5 cycles between Trial 1 (stopped after 5 cycles) and Trial 2 shows high thermal consistency. Trial 2 was conducted at a faster pace (10.7 minutes for 5 cycles vs. 12.5 minutes in Trial 1). The temperature rise (Delta T) is nearly identical: Winch A rose by 20.0°C in Try 1 vs. 18.0°C in Try 2; Winch C rose by 18.0°C in Try 1 vs. 20.0°C in Try 2; and Winch D rose by 11.0°C in Try 1 vs. 12.0°C in Try 2. This confirms a stable heat generation rate of 3.5°C to 4.0°C per cycle for the main hoist motors under a 20T load, proving that the thermal profile is highly repeatable.

# 6. Temperature Trend Plot