BMS BATTERY PERFORMANCE REPORT
Case Study: Operational State - HVAC ON

Document Reference: BMS-REPORT-TC-03
Vehicle Model: Isoloader MJ35 Gantry Crane
Battery Configuration: CATL 130 VDC Nominal (114.52 kWh)
System State: Operational (Active Duty Cycles)
HVAC System Status: ON (~3.37 kW)
Test Duration: 1799.91 s (30.00 mins)

# 1. Executive Summary

• Average Total Power: 1.3255 kW
• Total Energy Consumed during Test: 0.6627 kWh
• Projected Continuous Runtime (100% to 10% SOC): 77.76 Hours
• Test Verdict: PASS. The battery packs operate within normal parameters.

# 2. Statistical Analysis

| Metric | Pack A (BMSA) | Pack B (BMSB) | Combined Total |
| --- | --- | --- | --- |
| Voltage (Min) | 127.3 V | 127.3 V | - |
| Voltage (Max) | 127.6 V | 127.6 V | - |
| Voltage (Average) | 127.55 V | 127.56 V | - |
| Current (Min) | 2.600 A | 2.100 A | - |
| Current (Max) | 25.000 A | 25.300 A | - |
| Current (Average) | 5.415 A | 4.978 A | - |
| Power (Min) | 0.332 kW | 0.268 kW | 0.599 kW |
| Power (Max) | 3.185 kW | 3.221 kW | 6.378 kW |
| Power (Average) | 0.691 kW | 0.635 kW | 1.326 kW |

# 3. Data Plots and Visualization

## 3.1 Pack Voltages over Time

## 3.2 Pack Currents over Time

## 3.3 Power Consumption (kW)

## 3.4 Load Profile Distribution

## 3.5 State of Charge (SOC) Profile

# 4. Engineering Recommendations

1. Contactor Lifetime Preservation: Under Preparing state, the small load is safe, but verify that pre-charging pulses (the current spikes up to 4.5A) do not cause contactor degradation over time.

2. HVAC Impact: The air conditioning consumes about ~3.37 kW constantly. This represents a significant load during standby (representing about 91% of standby power). Operators should be trained to turn OFF the HVAC when the crane is parked in preparing state for long periods to conserve battery energy.

3. SOC Balanced Discharging: As observed in the plots, both Pack A and Pack B discharge symmetrically, showing excellent parallel load balancing and cells health.