BMS BATTERY PERFORMANCE REPORT
Case Study: Preparing State - HVAC OFF

Document Reference: BMS-REPORT-TC-01
Vehicle Model: Isoloader MJ35 Gantry Crane
Battery Configuration: CATL 130 VDC Nominal (114.52 kWh)
System State: Preparing (Init Stage 5)
HVAC System Status: OFF
Test Duration: 3599.96 s (60.00 mins)

# 1. Executive Summary

• Average Total Power: 0.3297 kW
• Total Energy Consumed during Test: 0.3297 kWh
• Projected Continuous Runtime (100% to 10% SOC): 312.64 Hours
• Test Verdict: PASS. The battery packs operate within normal parameters.

# 2. Statistical Analysis

| Metric | Pack A (BMSA) | Pack B (BMSB) | Combined Total |
| --- | --- | --- | --- |
| Voltage (Min) | 127.8 V | 127.8 V | - |
| Voltage (Max) | 127.9 V | 127.9 V | - |
| Voltage (Average) | 127.89 V | 127.90 V | - |
| Current (Min) | 0.800 A | 0.800 A | - |
| Current (Max) | 4.500 A | 4.600 A | - |
| Current (Average) | 1.253 A | 1.325 A | - |
| Power (Min) | 0.102 kW | 0.102 kW | 0.205 kW |
| Power (Max) | 0.576 kW | 0.588 kW | 1.164 kW |
| Power (Average) | 0.160 kW | 0.169 kW | 0.330 kW |

# 3. Data Plots and Visualization

## 3.1 Pack Voltages over Time

## 3.2 Pack Currents over Time

## 3.3 Power Consumption (kW)

## 3.4 Load Profile Distribution

## 3.5 State of Charge (SOC) Profile

# 4. Engineering Recommendations

1. Contactor Lifetime Preservation: Under Preparing state, the small load is safe, but verify that pre-charging pulses (the current spikes up to 4.5A) do not cause contactor degradation over time.

2. HVAC Impact: The air conditioning consumes about ~3.37 kW constantly. This represents a significant load during standby (representing about 91% of standby power). Operators should be trained to turn OFF the HVAC when the crane is parked in preparing state for long periods to conserve battery energy.

3. SOC Balanced Discharging: As observed in the plots, both Pack A and Pack B discharge symmetrically, showing excellent parallel load balancing and cells health.