BMS WINCH PERFORMANCE TEST REPORT
Validation of energy consumed, regenerated and motor temperatures under 20T load

Document Reference: BMS-VALIDATION-WINCH-01
Vehicle Model: Isoloader MJ35 Gantry Crane
Battery Configuration: CATL 130 VDC Nominal (114.52 kWh)
Test Configuration: Winch/Hoist Lifting (Nâng hạ tời 20 tấn), HVAC ON
Test Method: 5 continuous raise + lower cycles, lift height 4200 mm

# 1. Executive Energy & Speed Summary

• Total Energy Discharged (Gross): 1.6332 kWh
• Total Energy Regenerated: 0.7787 kWh
• Net Energy Consumed: 0.8545 kWh
• Regeneration Percentage: 47.68%

• Gross Energy Consumed per Cycle: 0.3266 kWh
• Regenerated Energy per Cycle: 0.1557 kWh
• Net Energy Consumed per Cycle: 0.1709 kWh

• 80% SOC Cycle Capacity (Gross): 280.5 Cycles (lượt lên xuống)
• 80% SOC Cycle Capacity (Net): 536.1 Cycles (lượt lên xuống)

• Maximum Hoisting Speed: 7.13 m/min (Raise) / 6.43 m/min (Lower)
• Average Hoisting Speed (overall stroke): 5.77 m/min (Raise) / 4.16 m/min (Lower)

# 2. Motor Balance & Temperature Analysis (Active Lifting)

| Parameter | Winch A (WinchA) | Winch B (WinchB) | Winch C (WinchC) | Winch D (WinchD) |
| --- | --- | --- | --- | --- |
| Average Current (A) | 75.71 A | 56.56 A | 73.97 A | 73.57 A |
| Maximum Current (A) | 98.00 A | 80.00 A | 94.00 A | 96.00 A |
| Mean Absolute Torque (Nm) | 33.59 Nm | 23.51 Nm | 33.09 Nm | 32.90 Nm |
| Mean Motor Temp (°C) | 59.4 °C | 0.0 °C | 58.9 °C | 52.2 °C |
| Maximum Motor Temp (°C) | 70.0 °C | 0.0 °C | 68.0 °C | 59.0 °C |

# 3. Data Plots and Visualization

## 3.1 Power Profile

## 3.2 Winch Drum Angular Position

## 3.3 Winch Motor Currents Comparison

## 3.4 Winch Motor Temperatures Comparison (Excluding WinchB)