BMS BATTERY & HVAC PERFORMANCE TESTS SUMMARY
Comparative Study for Isoloader MJ35 Gantry Crane

# 1. Comparative Executive Table

| Parameter / Metric | TC-01: Preparing (HVAC OFF) | TC-02: Preparing (HVAC ON) | TC-03: Operational (HVAC ON) |
| --- | --- | --- | --- |
| System State | Preparing | Preparing | Operational |
| HVAC Status | OFF | ON (~3.37 kW) | ON (~3.37 kW) |
| Average Total Power (kW) | 0.330 kW | 3.700 kW | 9.084 kW |
| Peak Total Power (kW) | 1.164 kW | 4.602 kW | 44.972 kW |
| Avg Current per Pack (A) | A: 1.25 A B: 1.32 A | A: 14.43 A B: 14.50 A | A: 35.14 A B: 35.89 A |
| Avg Voltage (V) | A: 127.89 V B: 127.90 V | A: 127.89 V B: 127.90 V | A: 127.90 V B: 127.90 V |
| Energy Consumption Rate | ~0.330 kWh/h | ~3.697 kWh/h | ~9.100 kWh/h |
| Usable Battery Capacity | 103.07 kWh | 103.07 kWh | 103.07 kWh |
| Projected Runtime | 312.6 Hours | 27.9 Hours | 11.3 Hours |

# 2. Key Findings & Comparisons

• Impact of HVAC: Activating the HVAC in the preparing state increases the power draw from 0.33 kW to 3.70 kW (an 11.2x increase). This reduces the standby battery runtime from 312.6 hours to 27.9 hours.

• Operational vs. Standby Power: In dynamic operational state with continuous duty cycles, the average power consumption is 9.10 kW (traction, hoist, HVAC). The projected runtime is 11.3 Hours, which satisfies the 8-hour shift requirement with a 29% safety margin.

• Regenerative Energy Recovery: Active energy regeneration returns up to -39 A per pack to the battery during lowering, recovering 15-25% energy and significantly extending operational runtime.