BMS BATTERY PERFORMANCE REPORT
Case Study: Operational State - HVAC ON

Document Reference: BMS-REPORT-TC-03
Vehicle Model: Isoloader MJ35 Gantry Crane
Battery Configuration: CATL 130 VDC Nominal (114.52 kWh)
System State: Operational (Active Duty Cycles)
HVAC System Status: ON (~3.37 kW)
Test Duration: 3599.00 s (59.98 mins)

# 1. Executive Summary

• Average Total Power: 9.0840 kW
• Total Energy Consumed during Test: 9.0801 kWh
• Projected Continuous Runtime (100% to 10% SOC): 11.35 Hours
• Test Verdict: PASS. The battery packs operate within normal parameters.

# 2. Statistical Analysis

| Metric | Pack A (BMSA) | Pack B (BMSB) | Combined Total |
| --- | --- | --- | --- |
| Voltage (Min) | 127.9 V | 127.9 V | - |
| Voltage (Max) | 127.9 V | 127.9 V | - |
| Voltage (Average) | 127.90 V | 127.90 V | - |
| Current (Min) | -29.827 A | -30.762 A | - |
| Current (Max) | 173.920 A | 177.865 A | - |
| Current (Average) | 35.136 A | 35.888 A | - |
| Power (Min) | -3.815 kW | -3.934 kW | -7.749 kW |
| Power (Max) | 22.244 kW | 22.749 kW | 44.972 kW |
| Power (Average) | 4.494 kW | 4.590 kW | 9.084 kW |

# 3. Data Plots and Visualization

## 3.1 Pack Voltages over Time

## 3.2 Pack Currents over Time

## 3.3 Power Consumption (kW)

## 3.4 Load Profile Distribution

## 3.5 State of Charge (SOC) Profile

# 4. Engineering Recommendations

1. Contactor Lifetime Preservation: Under Preparing state, the small load is safe, but verify that pre-charging pulses (the current spikes up to 4.5A) do not cause contactor degradation over time.

2. HVAC Impact: The air conditioning consumes about ~3.37 kW constantly. This represents a significant load during standby (representing about 91% of standby power). Operators should be trained to turn OFF the HVAC when the crane is parked in preparing state for long periods to conserve battery energy.

3. SOC Balanced Discharging: As observed in the plots, both Pack A and Pack B discharge symmetrically, showing excellent parallel load balancing and cells health.