BMS BATTERY PERFORMANCE REPORT
Case Study: Preparing State - HVAC ON

Document Reference: BMS-REPORT-TC-02
Vehicle Model: Isoloader MJ35 Gantry Crane
Battery Configuration: CATL 130 VDC Nominal (114.52 kWh)
System State: Preparing (Init Stage 5)
HVAC System Status: ON (~3.37 kW)
Test Duration: 3599.96 s (60.00 mins)

# 1. Executive Summary

• Average Total Power: 3.6997 kW
• Total Energy Consumed during Test: 3.6996 kWh
• Projected Continuous Runtime (100% to 10% SOC): 27.86 Hours
• Test Verdict: PASS. The battery packs operate within normal parameters.

# 2. Statistical Analysis

| Metric | Pack A (BMSA) | Pack B (BMSB) | Combined Total |
| --- | --- | --- | --- |
| Voltage (Min) | 127.8 V | 127.8 V | - |
| Voltage (Max) | 127.9 V | 127.9 V | - |
| Voltage (Average) | 127.89 V | 127.90 V | - |
| Current (Min) | 13.528 A | 13.633 A | - |
| Current (Max) | 18.141 A | 18.129 A | - |
| Current (Average) | 14.428 A | 14.499 A | - |
| Power (Min) | 1.730 kW | 1.744 kW | 3.504 kW |
| Power (Max) | 2.318 kW | 2.319 kW | 4.602 kW |
| Power (Average) | 1.845 kW | 1.854 kW | 3.700 kW |

# 3. Data Plots and Visualization

## 3.1 Pack Voltages over Time

## 3.2 Pack Currents over Time

## 3.3 Power Consumption (kW)

## 3.4 Load Profile Distribution

## 3.5 State of Charge (SOC) Profile

# 4. Engineering Recommendations

1. Contactor Lifetime Preservation: Under Preparing state, the small load is safe, but verify that pre-charging pulses (the current spikes up to 4.5A) do not cause contactor degradation over time.

2. HVAC Impact: The air conditioning consumes about ~3.37 kW constantly. This represents a significant load during standby (representing about 91% of standby power). Operators should be trained to turn OFF the HVAC when the crane is parked in preparing state for long periods to conserve battery energy.

3. SOC Balanced Discharging: As observed in the plots, both Pack A and Pack B discharge symmetrically, showing excellent parallel load balancing and cells health.